#

File: salora.info,  Node: Top,  Prev: (pioneer.info),  Next: (sanyo.info),  Up: (remotes.info)

******************************************************************************

SALORA

******************************************************************************

* Menu:

* Technical info::

* Cable decoder::

#

File: salora.info,  Node: Technical info,  Prev: Top,  Next: Cable decoder, Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:   12bits

+-+

Header:   | |

+ +--------------+..

580us

+-+

0:        | |

..+ +-----+..

200us

+-+

1:        | |

..+ +----------+..

400us

+-+

| |

After the last bit a ( ..+ +  ) is transmitted to terminate the last bit.

Space:    25000us

Code is transmited once and then xored with #11110b and then

transmited as long as a key is held down.

It seems that the HP cannot sample this form of codes.... :(

#

File: orion.info,  Node: Cable decoder, Prev: Technical info, Up: Top

******************************************************************************

CABLE DECODER

******************************************************************************

GROUP:  Not needed

Code    Function

----    --------

#A00    1

#A20    2

#A40    3

#A60    4

#A80    5

#AA0    6

#AC0    7

#AE0    8

#B00    9

#B20    0

#B40    1-

#B60    #

#B80    Ch-

#BA0    Ch+

#C60    Kan

#C40    MP

#CE0    M

#CC0    N

#8A0    fine tune -

#880    fine tune +

#840    power

#8C0    mute